PROCEDIMEINTOS NORTEL OPCION 11C

System software installation

Contents

This section contains information on the following topics:

Reference list . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 233

General information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 233

General information . . . . . . . . . . ... . . . . . . . . . . . . . . . . . . . . . . . . . . . . 243

Keycode Information . . . . . . . . . …... . . . . . . . . . . . . . . . . . . . . . . . . . . 233

How to install software in a new system . . . . …… . . . . . . .. . . . . . . . . 234

Software installation on the IP Expansion Cabinet using the

preprogrammed software daughterboard . . . . . . …... . . . . . . . . . . . . . . 244

Reference list

The following are the references in this section:

• Option 11C and 11C Mini Fault Clearing (553-3011-500)

• Maintenance (553-3001-511)

General information

This Chapter describes how to install software in a new Option 11C system.

Before continuing with the procedures outlined in this Chapter, verify that all

pertinent hardware is either connected to, or installed in the system.

Keycode Information

Data you need to enter during the software installation is provided on the

Keycode Datasheet.

If the keycodes are unsuccessful, check the following:

• software issue

• feature set name

• any additional packages

• TNs

• ISM parameters

• security ID

• auxiliary ID (the old site ID, if this is an upgrade)

• ensure the correct keycodes were entered. All items must match the

Keycode  Datasheet exactly.

For an Option 11C Mini system, check that the number of TNs has been

entered correctly from the keycode sheet.

When performing a new system installation, please ensure that the default

AUX ID matches the AUX ID from the Keycode Datasheet.

If they still are not successful, then call your Service Representative.

How to install software in a new system

Summary of steps

The Software Installation steps are summarized in the following list:

• Get MAC address from IP daughterboard

• Power up/Install IP Expansion cabinet

• Power up Main cabinet. Ensure link led is green

• Perform software installation on the Main cabinet

• Select the New System Installation menu

• Select Feature Set and packages

• Select a database

• Select Incremental Software Management (ISM) parameters.

Ensure Survivability parameter is configured correctly

• Configure MAC addrress of the IP Expansion cabinet

Option 11C Planning and Installation Guide

• Validate keycodes

• Set the system time and date

• Reboot the Main cabinet

• Wait for link to come up.

Note: IP expansion cabinet software must be the same software

version as the main Option 11C cabinet.

Procedure 38

Installing the software

Note: This procedure assumes that the system installation

procedures in Chapter 8 have been performed and that the system is

up and running. The Link LED on the software daughterboards should

be lit and green.

1.- Observe the terminal screen.

If the following is displayed:

SOFTWARE INSTALLATION PROGRAM

proceed to Step 3.

If the following is displayed:

Current  system time and date: 00:00:00 -- 00/00/00

proceed to Step 2.

If neither message appears, refer to the Option 11C and 11C Mini Fault

Clearing (553-3011-500).

2 .-Set the system time and date.

CAUTION

Wear the antistatic wrist strap provided in the bottom of the

cabinet before handling circuit cards. Static electricity can

damage the components of power supplies and circuit cards.

553-3021-210 Standard 7.00 January 2002

Note: The Time and Date prompt appears only when the Install Setup

Program detects a system Year Date that is not in the range of

1995-2095. The responses shown below are examples of how to enter

the system Time and Date:

Enter new time (hh/mm/ss)

08:00:00 <cr>

Enter new date (yy/mm/dd)

95/05/01 <cr>

08:00:00 -- 95/05/01 is the new system time and date

y <cr>

3.- Select Menu option 1 for a new installation.

Main Cabinet Software Installation Main Menu:

1. New Install or Option 11/11E Upgrade - From Software Daughterboard

2. System Upgrade

3. Utilities

4. New System Installation - From Software Delivery Card

[q]uit, [p]revious, [m]ain menu, [h]elp or [?], <cr> - redisplay

Enter selection: 1<cr>

Meridian 1 Software Rls (25.40) will be installed.

Note: The following prompts require information from the Keycode

data sheet. Please have it available.

4.- Select the Feature Set to be enabled.

Note: The Feature Set selected must match the ones provided with

key codes on the keycode data sheet. The Feature Set names shown

below are examples only.

Select Feature Set You Wish to Enable:

1. General Business (NTSKxxxx)

2. Enhanced Business (NTSKxxxx)

3. Enterprise (NTSKxxxx)

4. NAS/VNS (NTSKxxxx)

[q]uit, [p]revious, [m]ain menu, [h]elp or [?], <cr> redisplay

(example only:)

Enter Selection: 2 <cr> (Enhanced Business)

5.- Indicate whether or not packages are to be added.

Feature Set Selection: Enhanced Business

Do you wish to add packages?

Select no, yes or abort:

n <cr> (no)

y <cr> (yes)

a <cr> (abort)

Note: Abort returns you to the main menu.

If the response was NO go to Step 8 on page 238.

If the response was YES go to Step 6 on page 237.

6.- Select the Feature packages to be added.

Summary of Packages selected is:

0-2 4-5 7-14 16-25 28-29 32-64 67 70-77 79-83 86-93 95 98-104

107-111 113-116 118-120 122-125 127-129 131-133 135 137-141 167

Enter packages (s) to be added, blank line to end:

215-235 <cr>

Note: (<cr> ends selection entry or if no packages are to be added).

7.- Confirm Feature Set and packages.

Your Feature Set Selection is “Enhanced Business”:

Additional Packages selected: 215-235

Summary of Packages selected is:

0-2 4-5 7-14 16-25 28-29 32-64 67 70-77 79-83 86-93 95 100-104 107-111

113-116 118-120 122-125 127-129 131-133 135 137-141 167

...

...

215-235

Is this selection correct?

n <cr> (no)

y <cr> (yes)

a <cr> (abort, return to main menu)

If the response was NO go to Step 4 on page 236.

If the response was YES go to Step 8 on page 238.

8 Select a Database.

Select menu option 1 or 2 for a new system installation:

1. Pre-Configured database - Enhanced Business

2. Basic Configuration (Release 22)

3. CCBR Restore File

4. Option 11/11E Software Cartridge

[q]uit, [p]revious, [m]ain menu, [h]elp or [?], <cr> redisplay

Enter Selection: 1 <cr>.

9.- Confirm database selection.

Do you wish to continue?

n <cr> (no)

y <cr> (yes)

a <cr> (abort, return to main menu)

If the response was No, go to Step 8 on page 238.

If the response was YES, go to Step 10 on page 238.

10.- Review ISM parameters.

Note: On a new installation, the ISM parameters displayed on the

terminal screen will be the default settings associated with the Feature

Set selection. These settings can be accepted without changes or

changed to suit the requirements of the new system.

Example: Current ISM Parameters: 				Parameter Description

TNS 				(100)			 maximum number of terminal numbers

ACDN				 (300)			 maximum number of ACD DNs

AST				 (100)			 (maximum number of associate Sets

LTID 				(100)			 maximum number of Logical Terminal IDs

RAN CON 			(12)			 default RAN connection

RAN RTE 			(2500)			 default RAN routes

MUS CON 			(100) 			default MUS connection)

BRAND 				(0)			 brandline

ACD AGENTS 			(300)			 maximum number of ACD agents

ANALOGUE TELEPHONES	 (2500)			 maximum number of analogue sets

ATTENDANT CONSOLES		 (2500) 			maximum number of attendant sets

BRI DSL				 (100) 			maximum number of Digital Subscriber loops

CLASS TELEPHONES 		(2500)			 maximum number of class sets

DATA PORTS			 (2500)			 maximum number of data ports

DIGITAL TELEPHONES		 (2500) 			maximum number Digital sets

INTERNET TELEPHONES		 (0) 			maximum number of Internet sets

PHANTOM PORTS		 (2500) 			maximum number of phantom ports

WIRELESS TELEPHONES 		(2500) 			maximum number Wireless sets

WIRELESS VISITORS		 (0)			 maximum number Wireless Visitors

ITG ISDN TRUNKS 		(2500)			 maximum number ISDN trunks

TRADITIONAL TRUNKS 		(2500)			 maximum number of traditional trunks

TMDI D-CHANNELS 		(100)			 maximum number of channels

SURVIVABILITY			 (0)		 Number of Survivable IP expansion cabinets

Do you wish to change ISM parameters?

n <cr> (no change)

y <cr> (change)

a <cr> (abort, return to main menu)

If the response was YES go to Step 11 on page 239.

If the response was NO go to Step 12 on page 241.

11.- Select ISM parameters.

Do you wish to change any ISM parameters? (y/n/[a]bort): y

Enter new ISM parameters, <cr> to leave unchanged:

TNS		 		(100) 200

ACDN 				(300)

AST 				(100)

LTID 				(100)

RAN CON 			(12)

RAN RTE 			(2500)

MUS CON			(100)

BRAND 				(0)

ACD AGENTS 			(300)

ANALOGUE TELEPHONES 	(2500)

ATTENDANT CONSOLES 		(2500)

BRI DSL 			(100)

CLASS TELEPHONES		 (2500)

DATA PORTS 			(2500)

DIGITAL TELEPHONES 		(2500)

INTERNET TELEPHONES 		(0)

PHANTOM PORTS 		(2500)

WIRELESS TELEPHONES		 (2500)

WIRELESS VISITORS		 (0)

ITG ISDN TRUNKS 		(2500)

TRADITIONAL TRUNKS		 (2500)

TMDI D-CHANNELS 		(100)

SURVIVABILITY 		(0) 4

Confirm ISM parameters.

New ISM parameters are:

TNS 				(200)

ACDN 				(300)

AST				 (100)

LTID				 (100)

RAN CON			 (12)

RAN RTE 			(2500)

MUS CON 			(100)

BRAND				 (0)

ACD AGENTS			 (300)

ANALOGUE TELEPHONES 	(2500)

ATTENDANT CONSOLES		 (2500)

BRI DSL 			(100)

CLASS TELEPHONES 		(2500)

DATA PORTS			 (2500)

DIGITAL TELEPHONES		 (2500)

INTERNET TELEPHONES 		(0)

PHANTOM PORTS 		(2500)

WIRELESS TELEPHONES 		(2500)

WIRELESS VISITORS		 (0)

ITG ISDN TRUNKS 		(2500)

TRADITIONAL TRUNKS 		(2500)

TMDI D-CHANNELS		 (100)

SURVIVABILITY			 (4)

Is this correct?

n <cr> (no)

y <cr> (yes)

a <cr> (abort, return to main menu)

If the response was NO go to Step 10 on page 238.

If the response was YES go to Step 12 on page 241.

12.- Define the AUX ID.

Note: The default AUX ID is the security ID provided with the Option

11C. It should be replaced with the previous Option 11 or Option 11E

site ID.

Example:

Security ID: 10000326

Current AUX ID: 10000326

Do you wish to change the AUX ID?

y <cr> (yes)

n <cr> (no)

a <cr> (abort, return to main menu)

If the response was NO go to Step 14 on page 242.

If the response was YES go to Step 13 on page 241.

13.- Enter the AUX ID.

Enter the Option 11/11E Security ID for the new AUX ID,

<cr> to maintain

12121212 <cr>

New AUX ID: 12121212

Is this correct?

y <cr> (yes)

n <cr> (no)

a <cr> (abort, return to main menu)

If the response was NO go to Step 12 on page 241.

If the response was YES go to Step 14 on page 242.

14 .-Review and confirm information entered.

New Installation Information Summary:

Security ID : xxxxxxxx

Aux ID : xxxxxxxx

Cabinet Type : MAIN

Feature Set : Enhanced Business

Additional Pkgs : 215-235

Database : Company ABC

OLD  NEW

S/W Release : 2520F 2520X

ISM Parameters

TNS 							200 200

ACDN 							300 300

AST 							100 300

LTID							100 100

RAN CON 						12 12

RAN RTE 						2500 2500

MUS CON 						100 100

BRAND 							0 0

ACD AGENTS 						300 300

ANALOGUE TELEPHONES				 2500 2500

ATTENDANT CONSOLES 					2500 2500

BRI DSL 						100 100

CLASS TELEPHONES 					2500 2500

DATA PORTS 						2500 2500

DIGITAL TELEPHONES 					2500 2500

INTERNET TELEPHONES					 0 0

PHANTOM PORTS 					2500 2500

WIRELESS TELEPHONES 					2500 2500

WIRELESS VISITORS 					0 0

ITG ISDN TRUNKS 					2500 2500

TRADITIONAL TRUNKS 					2500 2500

TMDI D-CHANNELS 					100 100

SURVIVABILITY						4   4

Is this correct?

y <cr> (yes)

n <cr> (no)

a <cr> (abort, return to main menu)

If the response was NO go to Step 4 on page 236.

If the response was YES go to Step 15 on page 243.

15.- Enter the keycodes

Note: See Keycode Information on page 233 for important

information on Keycodes.

Enter new Keycodes:

Example:

Key 1:

Key 2:

Key 3:

xxxxxxxx <cr>

yyyyyyyy <cr>

zzzzzzzz <cr>

Keycode validation successful.

***WARNING*** A system restart will be invoked as part of the

software installation process.

If the successful message appears go to Step 16 on page 243.

If the unsuccessful message appears, repeat this step, Step 15 on page 243.

After three unsuccessful keycode validation attempts, the following

message appears:

Keycode validation unsuccessful.

Installation aborted….returning to main menu.

16 .-Complete the software installation.

Are you sure you wish to perform the installation?

y <cr> (yes)

n <cr> (no)

a <cr> (abort, return to main menu)

If the response was No go to Step 3 on page 236.

If the response was YES this is the end of the software installation

program. Continue with the next step, Step 17 on page 243.

17.- Observe the screen a second time once the installation program has

been completed.

Messages will appear on the TTY or terminal screen. When the

message “INIXXX” appears, the system is operational.

18.- Perform an EDD using LD 43. Refer to Maintenance (553-3001-511)

for further information on LD 43.

19.- Configure the IP Daughterboards’ MAC and IP addresses. Refer to

LD 117 in Maintenance (553-3001-511).

—————————— End of Procedure ——————————

Software installation on the IP Expansion Cabinet using the

preprogrammed software daughterboard

Point-to-Point or Layer 2 with bootp configuration

For Point-to-Point or Layer 2 with bootp configuration, you do not need a

TTY connected to the IP Expansion Cabinet. Power up the system, and the

software installs automatically.

Layer 2 or Layer 3 with manual configuration

IMPORTANT

BootP is a broadcast message used for IP Address discovery.

• For Point-to-Point installation, you must select Option 1.

• For Layer 2 LAN installation, the recommended selection is

Option 1.

• For Layer 3 LAN installation, you must select Option 2.

Procedure 39

Software installation for IP expansion, using the preprogrammed

software daughterboard

Note: This procedure is performed from a TTY connected to the IP

Expansion Cabinet

1 Power up the system, and the menu in Step 2 appears.

2 Select Manual Configuration (item 2) from the IP Expansion Cabinet

Configuration  Menu.

The IP Configuration Menu is displayed:

IP Configuration Menu:

1. Automatically Using BootP

2. Using Manual Configuration

3. Keep Existing Configuration

[q]uit, [p]revious, [m]ain menu, [h]elp or [?], <cr>

- redisplay

Enter Selection: 2

3.-Configure IP Expansion Parameters.

For the following menu, the sample IP parameters will be used:

IP address of the expansion cabinet 100BaseT(F): 47.147.20.101;

Subnet Mask of the expansion cabinet 100BaseT(F): 255.255.255.0;

Gateway address: 47.147.20.1;

IP address of the main cabinet 100BaseT(F): 47.147.10.100.

The IP Parameters menu is displayed:

Enter Expansion New IP Parameters:

Expansion IP: 47.147.20.101

Expansion NetMask: 255.255.255.0

Main IP: 47.147.10.100

Expansion Router/Gateway: 47.147.20.1

Is this correct? (t/n/[a]bort): Y

Note: ‘Expansion Router/Gateway’ appears only in a Layer 3

configuration.

4.- The software installation is completed automatically without user

intervention.

5 Refer to Software Input/Output Guide Maintenance (553-3001-511)

LD 117 to configure the IP Expansion cabinets IP Address.

—————————— End of Procedure ——————————

Connecting the telephones

Contents

This section contains information on the following topics:

Reference list . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 247

General information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 248

Cross-connecting telephones . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 249

Connecting telephones without a PFTU . .. . . . . . . . . . . . . . . . . . . . . . . 251

Connecting 500/2500-type telephones with a PFTU . . . . . . . . . . . . . . . 252

Connecting off-premise telephones . . . . . . . . . . . . . . . . . . . . . . . . . . . . 253

Connecting an attendant console . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 255

Cross-connecting terminal Digital Subscriber Loops . . . . . . . . . . . . . . 269

Activating telephones . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 269

Activating a default model with a character display . . . . . . . . . . . . . . . 270

Activating a default model without a character display . .. . . . . . . . . . . 272

Activating a customized model with a character display . .. . . . . . . . . . 273

Activating a customized model without a character display . . . . . . . . . 275

Activating terminals on a DSL . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 276

General information

This chapter contains instructions for connecting telephones to the

cross-connect terminal. It also contains the location of the individual

extension numbers (DNs) when you are implementing one of the default

numbering plans and how to activate each telephone.

Refer to the instructions provided with the telephone or console and to

Telephone and Attendant Console: Installation (553-3001-215) for detailed

information about installing telephones and consoles.

The cable from the cabinet card slot containing the line card associated with

the telephone being connected, must be installed before you continue. Refer

to “Installing and connecting cross-connect terminals to cabinets” on

page 169, if additional cabling is required.

Cable assignments

The cables from each cabinet are labeled J1 through J10 at the cross-connect

terminal. Each cable represents a specific set of Terminal Numbers (TNs) as

shown in Table 37 on page 258.

WARNING

Always use caution when installing or modifying telephone lines.

Avoid installing telephone wiring during a lightning storm. Do not

install telephone jacks in wet locations unless the jack is specifically

designed for wet locations. Never touch uninsulated telephone wiring

unless the line has been disconnected at the network interface.

Cross-connecting telephones

Connect the telephones according to the following figures. Cross-connections

for Analog (500/2500 type) telephones are shown in Figure 94 on page 250,

and for Meridian Digital Telephones in Figure 95 on page 251.

Procedure 40

Cross-connecting telephones

1.- Locate telephone terminations at the cross-connect terminal.

2.- Connect Z-type cross-connecting wire to the leads of the

telephone.

3.- Locate line circuit card (TN) terminations at the cross-connect

terminal.

4.- Connect the other end of the cross-connecting wire to the

assigned TN terminal block.

Figure 94

NE-500/2500-type telephone  cross-connections

Figure 95

Meridian Digital Telephone cross-connections

Connecting telephones without a PFTU

Procedure 41

Connecting telephones without a PFTU

1.- Locate the telephone terminations on the cross-connect terminal.

2.- Connect one end of the cross-connect wire to the leads of the

Telephone

3.- Locate the line card terminations on the cross-connect terminal.

Refer to Table 37 on page 258 and Table 38 on page 259.

4.- Connect the other end of the cross-connect wire to the assigned

TN terminal block.

Default DN assignments are given in the following Tables:

• Table 42, “Main Cabinet Default DN assignments,” on

page 263

• Table 43, “Expansion Cabinet 1 Default DN assignments,” on

page 264

• Table 44, “Expansion Cabinet 2 Default DN assignments,” on

page 265

• Table 45, “Expansion Cabinet 3 Default DN assignments,” on

page 267

• Table 46, “Expansion Cabinet 4 Default DN assignments,” on

page 268

The telephone can now be activated as described in “Activating

telephones” on page 269.

Note: All the following tables show the default DNs.

Connecting 500/2500-type telephones with a PFTU

Procedure 42

Connecting 500/2500-type telephones with a PFTU

1 .-	Locate the telephone terminations on the cross-connect terminal.

2.-	 Connect one end of the cross-connect wire to the leads of the

telephone.

3 .-	Locate the PFTU connections (unit PFT 1 through PFT 5)

assigned to this telephone at the cross-connect terminal.

Refer to Table 39 on page 261 and Table 40 on page 262.

4.-	 Connect the other end of the cross-connect wire to the pair

assigned to the telephone on the PFTU.

5 .-	Connect a second cross-connect wire to the pair assigned to the

line card on the PFTU.

6.-	 Locate the line card terminations on the cross-connect terminal.

Refer to Table 37 on page 258 for the Main cabinet and Expansion

cabinets 1 and 2.

Refer to Table 38 on page 259 for Expansion cabinets 3 and 4.

7.-	 Connect the other end of the cross-connect wire to the assigned

TN terminal block

Default DN assignments are given in the following Tables:

• Table 42, “Main Cabinet Default DN assignments,” on

page 263

• Table 43, “Expansion Cabinet 1 Default DN assignments,” on

page 264

• Table 44, “Expansion Cabinet 2 Default DN assignments,” on

page 265

• Table 45, “Expansion Cabinet 3 Default DN assignments,” on

page 267

• Table 46, “Expansion Cabinet 4 Default DN assignments,” on

page 268

The telephone can now be activated as described in “Activating

telephones” on page 269.

Connecting off-premise telephones

Off-premise 500/2500-type telephones must be connected through an

NTAK92AA Off-Premise Protection module. Each module can connect up to

four analog telephones and can interface with an NT1R20 Off-Premise

Station analog Line Card, an NT8D03 Analog Line Card or with an NT8D09

Message Waiting Line Card.

WARNING

The message waiting line card produces -150 volts which is considered

hazardous on off-premise telephones. Make sure that the -150 V is

disabled on off-premise telephones.

The voltage is disabled when the Class Of Service (CLS) assigned to

the telephone is LPD (message waiting lamp denied) and MWD

(Message Waiting Denied) in LD 10.

Refer to Software Input/Output Guide Maintenance (553-3001-511)

for information about LD 10.

Under no circumstances shall LPA or MWA be assigned in the Class of

Service on off-premise telephones

Procedure 43

Connecting an off-premise telephone

1	 Mount the NTAK92AA Off-Premise protection module on the wall

using four #10 1/2 in (minimum) screws.

2 	Connect a #6 AWG (#40 Metric Wire Gauge) from the ground lug

at the bottom of the NTAK92AA Off-Premise protection module to

an earth ground. Refer to Figure 96 on page 256.

WARNING

If connecting to a message waiting line card, unseat the card from its

assigned slot before proceeding with the next step

3 	Connect two NTAK9204 cables (one from connector J1 and one

from connector J2) from the protection module to the

cross-connect terminal.

Terminate the cables as shown in Figure 94 on page 250.

4 	Cross-connect the J1 cable to the Tip and Ring connections

coming from the line card.

Default DN assignments are given in the following Tables:

• Table 42, “Main Cabinet Default DN assignments,” on

page 263

• Table 43, “Expansion Cabinet 1 Default DN assignments,” on

page 264

• Table 44, “Expansion Cabinet 2 Default DN assignments,” on

page 265

• Table 45, “Expansion Cabinet 3 Default DN assignments,” on

page 267

• Table 46, “Expansion Cabinet 4 Default DN assignments,” on

page 268

5 	Cross-connect the J2 cable to the off-premise telephone.

6	 Install the regulatory label provided with the Off-Premise

protection module on the inside right-hand wall of the cabinet

near the velcro wriststrap fastener.

7	 Install the line card in its assigned position.

The telephone can now be activated as described in “Activating

telephones” on page 269

Connecting an attendant console

Procedure 44

Connecting attendant console

1 	Locate the attendant console terminations at the cross-connect

terminal.

2 	Locate the line card terminations at the cross-connect terminal.

3 	With cross-connect wire, connect the line card and other

connections to the console as shown in Figure 97 on page 257.

Figure 96

NTAK92AA Off-Premise Protection Module connections

Figure 97

Attendant console connections

Cross-connecting terminal Digital Subscriber Loops

Refer to Option 11C ISDN BRI Hardware Installation and Maintenance

(553-3011-311), for a complete description of terminal Digital Subscriber

Loops (DSL) cross-connecting and installation.

Activating telephones

Activating telephones is straightforward. You activate each telephone by

carrying out a procedure on the telephone itself. The software feature used to

activate telephone sets is called “Automatic Set Configuration.”

Note: The data feature cannot be activated using the procedures in this

chapter. If you want to program a telephone with the data feature, you

must manually program it in LD 11 with Data class of service as

described in Software Input/Output Guide Maintenance

(553-3001-511).

Procedures for activating the following models of telephones are outlined:

• a default model with default extension number

• a customized model with a customized extension number.

These procedures are presented for telephones with and without character

displays.

Telephone tones

There are a number of different telephone tones. The following table includes

tones that you will hear during telephone activation:

Note 1: Before activating a telephone, make sure that you have decided

on its final location and are aware of the model number assigned to it and

if it is to be customized.

Note 2: When activating Meridian Digital telephones, remember that

they differ from the older Meridian Modular telephones. Meridian

Modular telephones are allowed a combined total of 128 model

telephones, whereas Meridian Digital telephones are allowed 128 models

for each type of telephone. When you activate a Meridian Digital

telephone, you must select the model associated with that telephone type

or the telephone will not work.

Activating a default model with a character display

Procedure 45

Activating the telephone

1 	Plug the telephone set into the jack and wait 20 seconds before

picking up the handset. If you do not receive a dial tone, replace

the handset and wait another 10 seconds before going off-hook

again. Repeat this procedure until dial tone is received.

If successful, the character display shows either “MODEL? X” (if the

telephone relocation feature is not in use) or “RELOC OR MODEL? X”

(if the telephone relocation feature is in use). “X” represents the default

model for the telephone that you are activating.

Note 1: If you do not see the prompt “MODEL X” after lifting the

handset, disconnect the telephone from the wall jack, wait five

seconds, and re-insert the telephone into the jack. The telephone now

shows “MODEL X” when you lift the handset.

Note 2: The 20 second time interval is required for the system to

determine whether the set is new or if it is being relocated using the

Modular Telephone Relocation feature.

2 	Press the pound key (#) to select the default model.

The character display shows “OK, EXTENSION? XXXX”. “XXXX”

represents the default extension number for this telephone type.

3	 Press the pound key again to select the default extension

number.

You hear the relocation tone. The character display, shows “OK”.

OR

If the extension number is already in use by another telephone, you

hear the special dial tone. If the telephone has a display it shows

“MULTI-LINE, EXTENSION?”.

To accept the default extension number press the pound key.

To select a new extension number, manually enter an extension

number and press the pound key.

OR

If the extension number is not available for use, the character display

shows “ERROR, EXTENSION?“ and you hear overflow tone.

This happens when you choose an extension number manually or

when extension numbers are entered for additional keys. A default

extension number will not be offered if it is not available.

Step 3 must be repeated and you must manually enter a new

extension number.

Note 1: If other keys require secondary extension numbers, you are

prompted until you enter all of the required extension numbers for the

model.

Note 2: These extension numbers cannot be defaulted. The text

display prompting for further extension numbers is “KEY kk EXT?”

where “kk” represents the key number requiring the extension number.

Page 272 of 408 Connecting the telephones

Note 3: Each prompt for another extension number is accompanied

by special dial tone. When you are programming an extension number,

the lamp associated with that number on the telephone is lit.

4 	Hang up the telephone receiver.

After approximately 10 seconds, the telephone is configured.

Note: If you replace the handset before completing the prompt

sequence, the installation will automatically fail. This can be useful if

you make an error and want to restart the procedure.

Activating a default model without a character display

Procedure 46

Activating the telephone

1 	Plug the telephone set into the jack and wait 20 seconds before

picking up the handset. If you do not receive dial tone, replace the

handset and wait another 10 seconds before going off-hook

again. Repeat this procedure until dial tone is received.

Note: The 20 second time interval is required for the system to

determine whether the set is new or if it is being relocated using the

Modular Telephone Relocation feature.

2	 Press the pound key (#) to select the default model.

3 	Press the pound key again to select the default extension

number.

You hear a short, high-pitched beep lasting four seconds followed by

silence (relocation tone).

Note: If the extension number is already in use by another telephone,

you hear three beeps followed by continuous dial tone (special dial

tone).

To accept the default extension number press the pound key.

To select a new extension number, manually enter an extension

number and press the pound key.

If the extension number is not available for use, you hear a fast,

high-pitched broken tone (overflow tone). (This happens when you

choose an extension number manually or when extension numbers

are entered for additional keys. A default extension number will not be

offered if it is not available). Step 3 must be repeated and you must

manually enter a new extension number.

Note: If other keys require secondary extension numbers, you are

prompted until you enter all of the required extension numbers for the

model. These extension numbers cannot be defaulted. You are

prompted for each additional extension number with three beeps

followed by continuous dial tone (special dial tone). When you are

programming an extension number, the lamp associated with that

number on the telephone is lit.

Activating a customized model with a character display

Procedure 47

Activating a customized telephone

1 	Plug the telephone set into the jack and wait 20 seconds before

picking up the handset. If you do not receive dial tone, replace the

handset and wait another 10 seconds before going off-hook

again. Repeat this procedure until dial tone is received.

If successful, the character display shows either “MODEL? X” (if the

telephone relocation feature is not in use) or “RELOC OR MODEL? X”

(if the telephone relocation feature is in use). “X” represents the default

model for the telephone that you are activating.

Note 1: If you do not see the prompt “MODEL X” after lifting the

handset, disconnect the telephone from the wall jack, wait five

seconds, and re-insert the telephone into the jack. The telephone now

shows “MODEL X” when you lift the handset.

Note 2: The 20 second time interval is required for the system to

determine whether the set is new or if it is being relocated using the

Modular Telephone Relocation feature.

2 	Press the digits associated with the customized model and press

the pound key (#).

Dial tone disappears after the first digit is pressed and you hear special

dial tone after you press the pound key. If you enter a valid model

number the character display reads “OK, EXTENSION?”. If you enter

an invalid model, the previous prompt is reissued and you hear

overflow tone.

3 	Enter the customized extension number and press the pound

key.

You hear relocation tone. The character display shows “OK”.

OR

If the extension number is already in use by another telephone, you

hear special dial tone again. The character display shows

“MULTI-LINE, EXTENSION?”.

OR

If the extension number is not available for use, you hear overflow

tone. The character display shows “ERROR, EXTENSION?” and you

must repeat this step.

Note: If other keys require secondary extension numbers, you are

prompted until you enter all of the required extension numbers for the

model. These extension numbers cannot be defaulted. The text

display prompting for further extension numbers is “KEY kk EXT?”

where “kk” represents the key number requiring the extension number.

Each prompt for another extension number is accompanied by special

dial tone. When you are programming an extension number, the lamp

associated with that number on the telephone is lit.

4 	Hang up the telephone handset.

After approximately 10 seconds, the telephone is configured.

Note: If you replace the handset before you complete the prompt

sequence, the installation will automatically fail. This can be useful if

you make an error and want to restart the procedure.

Connecting the

Activating a customized model without a character display

Procedure 48

Activating a customized telephone

1 	Plug the telephone set into the jack and wait 20 seconds before

picking up the handset. If you do not receive dial tone, replace the

handset and wait another 10 seconds before going off-hook

again. Repeat this procedure until dial tone is received.

Note: The 20 second time interval is required for the system to

determine whether the set is new or if it is being relocated using the

Modular Telephone Relocation feature.

2	 Press the digits associated with the customized model and press

the pound key (#).

Dial tone disappears after the first digit is pressed, and you hear three

beeps followed by continuous dial tone (special dial tone) after you

press the pound key.

3 	Enter the customized extension number and press the pound

key.

You hear a short high-pitched beep lasting four seconds, followed by

silence (relocation tone).

If the extension number is already in use by another telephone, you

hear special dial tone again.

If the extension number is not available for use, you hear a fast,

high-pitched broken tone (overflow tone), and you must repeat this

step.

4 	Hang up the telephone handset.

After approximately 10 seconds, the telephone is configured.

Note: If other keys require secondary extension numbers, you are

prompted until you enter all of the required extension numbers for the

model. These extension numbers cannot be defaulted. You are

prompted for each additional extension number with three beeps

followed by continuous dial tone (special dial tone). When you are

programming an extension number, the lamp associated with that

number on the telephone is lit.

Activating terminals on a DSL

Refer to Option 11C ISDN BRI Hardware Installation and Maintenance

(553-3011-311), for information about activating and initializing the various

terminals that can be connected to a terminal DSL.

Connecting the trunks

Contents

This section contains information on the following topics:

Reference list . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 278

General information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 278

Connecting trunks without PFTU . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 279

Connecting trunks with PFTU . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 279

Trunk Connections . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 282

NT8D14 Universal trunk card . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 282

NT8D15 E&M Trunk card . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 284

NT6D70 SILC and NT6D71 UILC cards . . . . . . . . . . . . . . . . . . . . . . . 286

Trunk connections (Europe) . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 286

E&M TIE trunk card (2-Wire) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 286

E&M TIE trunk card (4-Wire) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 288

E&M TIE trunk card (2280Hz) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 291

E&M TIE trunk card (RAN) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 292

E&M TIE trunk card (MUS) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 292

CO & DID trunk card . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 293

Central Office trunk card . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 295

Trunk connections (UK) . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 296

NT5K17 Direct Inward Dial card terminations . . . . . . . . . . . . . . . . . . . 296

NT5K18 Exchange line trunk card terminations . . . . . . . . . . . . . . . . . . 297

NT5K19 Analog TIE line trunk card terminations . . . . . . . . . . . . . . . . 299

Activating a default model trunk . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 303

Activating a selected model trunk . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 305

General information

This Chapter describes how to connect trunks directly to the trunk card, with

or without the use of a Power Fail Transfer Unit (PFTU).

Note: The QUA6 Power Fail Transfer Unit operates with loop-start and

ground-start CO trunks. However, with ground-start trunks the

associated telephone set must be equipped with a ground-start button.

On the Option 11C, during the initial software installation, the installer has

the option of loading a default database containing pre-programmed trunk

data into software. If necessary, the default data can be modified at any time

to meet the specific needs of a customer. For a complete description of how

to modify pre-programmed trunking data, refer to “Changing

pre-programmed data” on page 387 in this Guide.

Connecting trunks without PFTU

Procedure 49

Connecting trunks without PFTU

1 	From the assignment record, determine the location of the trunk

connection and its associated Terminal Number (TN) at the

cross-connect terminal.

2 	With cross-connecting wire, connect the trunk to the TN.

Make sure that the wiring is not reversed and is on the proper

terminals.

The connections for trunks are listed in Table 49 on page 282,

Table 50 on page 283 and Table 51 on page 284.

Connecting trunks with PFTU

Procedure 50

Connecting trunks with PFTU

1 	Locate the PFTU terminal blocks at the cross-connect terminal.

2 	Cross-connect the first pair of the assigned PFT to the telephone.

Refer to Table 48 on page 280 for PFTU connections.

3 	Cross-connect the second pair of the PFT to the TN assigned to

the telephone.

4	 Cross-connect the third pair of the PFT to the central office trunk.

5 	Cross-connect the third pair of the PFT to the TN assigned to the

trunk.

6	 Repeat for each trunk assigned to the PFTU.

Trunk Connections

NT8D14 Universal trunk card

The universal trunk card provides eight analog trunks which can function in

the modes shown in Table 49 on page 282.

NT6D70 SILC and NT6D71 UILC cards

Refer to Option 11C ISDN BRI Hardware Installation and Maintenance

(553-3011-311) for a complete description of trunk DSL installation and

connections.

Trunk connections (Europe)

E&M TIE trunk card (2-Wire)

Note: Refer to the Intelligent Peripheral Equipment Supplements for a

complete description of European circuit cards.

NT5K18 Exchange line trunk card terminations

Cross connect the NT5K18 Exchange line trunk card as shown in Table 62.

Note: The connections on the NT5K18 Exchange line trunk card are

polarity sensitive. Make sure the ground side of the trunk is connected to

the A leg of the NT5K18 circuit. Make sure the -50 volt side of the trunk

is connected to the B leg of the NT5K18 circuit.

Activating a default model trunk

Due to the automatic thirty second time-out on the administration menu, be

sure that you have prepared the data that you want to input before you begin.

To determine corresponding TNs and trunks, check the location of trunk

cards in the cabinet or use LD 32.

Procedure 51

Activating a default model trunk

1	Pick up the handset of the administration telephone.

2 	Enter the Option 11C administration Flexible Feature Code to

access the administration menu.

The prompt “PASSWORD?” appears.

3 	Enter the default administration telephone password.

You hear special dial tone, and the prompt

“TASK?”

appears in the top line of the character display.

The second line of the display reads “1 ADD TRUNK”.

4	Select “1 ADD TRUNK” by entering the number “1”.

The prompt “ROUTE ACCESS?” appears on the character display.

5 	Enter the access code of the route to which you want to add a

trunk and press the pound key (#).

The prompt

“TN?”

asks you to enter a TN (Terminal Number) from one of the installed

trunk cards.

If you do not enter a valid route number, “TN?” does not appear and

the screen remains the same. If the type of trunk card does not match

the route, the prompt “ROUTE ACCESS?” appears again, and you

hear overflow tone.

6 	Enter the TN in Option 11 format (CCUU) and press the pound

key.

The prompt

“MODEL”

appears. If you enter an invalid TN, the display shows “INVALID, TN?”

and you must enter a new TN using the Option 11 format.

7 	Press the digits to select a trunk model (as assigned in LD 16).

The character display shows

“OK”.

After a delay of approximately 4 seconds you hear special dial tone.

The sequence is repeated when the prompt

“TN?”

appears on the character display.

The next valid trunk TN is automatically incremented after each trunk

is activated.

8 	Terminate the sequence by hanging up the telephone receiver, or

Repeat the sequence by going through the steps again.

By entering “#” when the procedure repeats, you accept the next TN

and are prompted for the model type.

By entering “#” again, you accept the previously accepted model.

Note: The model chosen during the first trunk activation sequence will

be the default model for all subsequent trunks until you hang up the

telephone or manually enter a new trunk model number.

Activating a selected model trunk

Due to the automatic thirty second time-out on the administration menu, be

sure that you have prepared the data that you want to input before you begin.

To determine corresponding TNs and trunks, check the location of trunk

cards in the cabinet or use LD 32.

Procedure 52

Activating a selected model trunk

1 	Pick up the handset of the administration telephone.

2 	Enter the administration Flexible Feature Code to access the

administration menu.

3 	Enter the default administration telephone password.

You hear special dial tone and the prompt “TASK?” appears on the top

line of the character display. The second line of the character display

reads “1 ADD TRUNK”.

4 	Select “1 ADD TRUNK” by entering the number “1”.

The prompt

“ROUTE ACCESS?”

appears.

5 	Enter the access code of the configured trunk route to which you

want to add the trunk and press the pound key (#).

The prompt

“TN?”

asks you to enter a TN from one of the installed trunk cards. If you do

not enter a valid route number, “TN?” does not appear and the screen

remains the same. If the card does not match the route, the prompt

“ROUTE ACCESS?” appears again, and you hear overflow tone.

6 	Enter the TN in Option 11 format (CCUU).

The response

“MODEL?”

prompts you to select a model number for the trunk.

7	 Enter a trunk model number for the specified TN and route and

press the pound key.

The character display shows:

“OK”

After a delay of approximately 4 seconds you hear special dial tone

and the sequence is repeated when the following prompt appears on

the character display.

“TN?”

8 	Hang up or repeat the sequence.

The sequence ends when the last unit in the card is used and the

program is complete, or when you hang up the telephone receiver.

Note: The model used for the first trunk activated in the sequence will

be the default for all subsequent trunks until the telephone is hung up

unless you manually enter a new trunk model number.

Connecting an external alarm

Contents

This section contains information on the following topics:

General information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 307

Alarm port assigned in software . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 307

Alarm through a QUA6 PFTU . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 308

General information

There are two methods of connecting an external alarm to the system:

• 	through an alarm port assigned in software

• 	through contacts in a QUA6 Power Fail Transfer Unit (PFTU).

Alarm port assigned in software

The system can be equipped with an alarm port using an analog line

connected to an Analog (2500/500)-type telephone or other similar type of

ringing or alerting device.

The alarm will operate when a BSD090 message indicates a power fault in

the system. Information about BSD090 messages is contained in

Maintenance (553-3001-511).

Procedure 53

Installing an alarm using an alarm port

1	 Install an analog (500/2500)-type line as described in “Connecting

the telephones” on page 247.

2	 Install an analog (500/2500)-type telephone or other similar

alerting device used as an alarm to the line.

The set can be typically assigned as a Model 20.

3 	Use LD 15 and make the following changes. Only the prompts

requiring a response are listed. Press return in response to the

other prompts.

- Enter CHG in response to the prompt REQ

- Enter CDB in response to the prompt TYPE

- Enter the customer number (0-99) in response to the prompt

CUST

• 	Enter the DN of the line assigned as an alarm port in response

To the prompt ALDN.

Note: If the assigned DN is inadvertently called the alarm will activate.

To avoid false alarms, make sure that the DN is not consistent with the

assigned numbering.

• 	Press return in response to the remaining prompts.

Alarm through a QUA6 PFTU

A QUA6 PFTU can be used to connect an external alarm through normally

open or normally closed contacts of one of its units. The contacts will operate

under the same conditions as the PFTU itself, and can support the following

capacities:

General information

When an Option 11C system is initially installed, customer data must be

entered into the overlay programs. Telephones, for example, must be

assigned features on their keys to allow them to function properly.

However, the Software Daughterboard can be pre-programmed with the

customer data. If you load pre-programmed data into the system during the

Installation process, some overlay entries will be automatically configured onthe telephones.

For example, you can choose a telephone model that has

predetermined feature and key assignments and a preassigned class of

service.

This can be a significant time-saver if you have to program numerous

types of telephone models.

Pre-programmed data is not mandatory for software installation. In fact, the

Software Daughterboard can be programmed with the minimum number of

files to allow the Option 11C to operate.

Passwords and codes

The following table shows each function in the left column and a

corresponding password or code on the right.

Default numbering plan

The default numbering plan for Option 11C is based on the following

guidelines:

• 	The default numbering plan uses four digits and starts at 2200.

• 	The prime extension number (DN) for each telephone is in the range

2200-2XXX.

The value of “XXX” varies depending on the number of

telephones that you have in the system. Any secondary extension

numbers use numbers outside this range.

This arrangement allows

Option 11 and Meridian Mail to automatically configure telephones and

corresponding mailboxes without manual intervention.

• 	Meridian Mail uses extension 7000 for access, 7001 for Auto Attendant,

7002 for Express Messaging, and 7003 for Prompt Maintenance.

• 	Meridian Mail Virtual Agents are pre-configured for Card 10 in Unit 0,

1, 2, 8, 9, and 10 of the main cabinet.

• 	The Central Answering Position has an extension of 7700. This

extension can be changed in LD 15.

First digits

Figure 71 on page 316 shows the default numbering plan for Option 11C:

The digit “7” in the default numbering plan is programmed with many system

features to assist you in configuring the Option 11C system. If you have the

Meridian Mail card option software, it automatically configures user mail

boxes to correspond with the 2200 numbering plan. In addition, the

pre-programmed Meridian Mail queues in the Meridian Mail software match

the default data on the Option 11C system

Flexible Feature Codes

Flexible Feature Code (FFC) data is used in many administrative procedures.

The table below lists the FFCs for the Option 11C system.

SDI ports

The minimum system port configuration for Option 11C is 3 SDI ports, all of

which can be found on the NTDK20 System Core card.

You can obtain additional ports by installing an NTAK02 SDI/DCH circuit

card. The SDI/DCH card has 4 serial I/O ports with three possible

configurations:

• 	2 asynchronous ESDI ports with 2 DCHI ports

- 2 synchronous ESDI ports, or

• 	2 SDI ports.

The default SDI port configuration is shown below. The value for “XX” is set

on the faceplate of the CPU/CONF circuit card

Administration and maintenance terminal port

Option 11E has two pre-configured maintenance ports on the NTBK45

System core card: port 1 and port 2.

Modem port

The pre-configured modem port allows the remote maintenance modem to be

connected with further system programming. This port is pre-configured as

TTY 0 (port 0 on the System core card) and is programmed for Maintenance

(MTC), Service Change (SCH) and BUG messages.

Enhanced Serial Data Interface (ESDI) port

Port 9 is pre-programmed as an ESDI port and supports Meridian Mail. It

functions as a Command Status Link with the settings given in Table 80:

Telephone tones

The telephone tones in North America are as follows:

Dial tone – A continuous tone.

Special dial tone – Three beeps followed by continuous dial tone.

Overflow tone – Like a busy tone, except faster and higher.

Relocation tone – A short high-pitched beep lasting for 4 seconds, followed

by silence.

Trunk routes

Table 81 on page 329 shows pre-programmed trunk route information that

you need on hand to activate and modify trunks.

follows:

CDR YES

INC YES

OAL YES

AIA YES

Trunk models

Note: All trunks are programmed as immediate start / supervision = yes,

with the exception of trunks with an asterisk beside them (*). Trunks

marked with an asterisk (*) are set for wink start / supervision = yes.

Model telephones

Note: OPS telephones do not have their own telephone models. You

can, however, create OPS models in by entering DD in response to the

CDEN prompt in LD 10.

With the exception of the administration and CAP model telephones, the

models are organized according to software packages, as follows:

Administration telephones

• 	M2008 - one model

• 	M2616 - one model

CAP telephones

- M2616 - one model

- M2216 - one model

General Business telephones

• 	Analog (500/2500 type) telephones - seven models

- M2000 telephones

— M2317 - two models

— M2009 - two models

— M2112 - two models

— M2018 - two models

• 	M2006 - five models

• 	M2008 - fifteen models

• 	M2616 - twelve models

- M2216 - two models

• 	M3000 - two models.

Hotel and Motel telephones

• 	500- and 2500-type telephones - one model

- M2006 - two models

- M2008 - two models

• 	M2616 - one model

Health Care telephones

- M2006 - one model

• 	M2008 - four models

• 	M2616 - two models.

This section provides model numbers for each telephone type and tables

showing the numbering and placement of the keys, and the feature assigned

to each key.

Acronyms for Class of Service options are shown in the following tables for

each model telephone in this section.

Administration telephones

M2008 model 99

Intended use: Administration (maintenance) telephone

Assigned Class of Service options:

MTA/ADD/LNA/FNA/GPUA/MWA/FBD

Central Answering Position (CAP) model telephones

M2616 and 2216 CAP telephone model 60

Intended use: Central Answering Position telephone

Assigned Class of Service options:

LNA/FND/GPUA/MWA/ADD/HFD/AGN/FBD/SPV

* This key is used as a Hotline to connect to the office paging system. Assign

it with the paging route access code and define it when you activate the

telephone.

Note: With the M2616 CAP you must use an ACD character display. In

the system software, the Extend key is actually called a Transfer key, the

Consult/Join key is called a Conference key, and the Add key is called a

No Hold Conference key.

General business models

M2000 series telephones

The M2000 series telephones - M2112, M2018, M2317, M2009, and

M3000 - are limited to a combined total of 128 model telephones. (In other

words, there can be no more than a combined total of 128 different models for

all the M2000 series telephones.)

It is up to you to make sure that the model associated with the telephone you

are activating is pre-defined. You must select the model associated with the

telephone or the telephone will not work.

M2317 model 20

Intended use: Two-line general business telephone with display

Assigned Class of Service options: FND/GPUA/LND/MWD/ADD/FBD

M2112 model 22

Intended use: Two-line general business telephone

Assigned Class of Service options: FND/GPUA/LNA/MWA/NDD/FBD

Analog (500/2500-type) telephone models

Note: When you are activating an analog (500-type) telephone, you

must use an analog (2500-type) telephone to define the features. When

you have done this, disconnect the analog (2500-type) telephone and

replace it with the analog (500-type) telephone that you are

activating.Analog (500/2500-type) support personnel telephone models.

Hotel and Motel models

Analog 500- and 2500-type telephones

Note: When you are activating a 500-type telephone, you must use a

2500-type telephone to define the features. Once you have done this,

unplug the 2500-type telephone and replace it with the 500-type

telephone that you are activating.

General information

The pre-programmed data on the Option 11C system can provide an effective

starting point for programming the system’s telephone and trunking

information.

This chapter shows how to change the default Option 11C numbering plan.

You may want to change the numbering plan for one or more of the following

reasons:

- to change the first number in the numbering plan

- to shift the start of the numbering plan to another card slot

- the default numbering plan interferes with the system data.

In addition to making changes to the default numbering plan, this section

provides information about modifying model telephone and trunk

programming stored in the Option 11C pre-programmed data

Changing the default numbering plan

For improved performance refer to the Option 11C Survivability Guide

(P0919734) for numbering plan configuration.

Extensions assigned differ from the default numbering plan

Compare the first digits assigned to this system with the default numbering

plan as shown in the “Default numbering plan” .

• If the first digit is in the unused range, change the first number in the

default numbering plan. To do this, follow the instructions in the section

of this chapter called io.

• If the first digit is being used by Meridian Mail, trunk routes, or other

data, you must remove these interferences. Follow the instructions found

later in this chapter in the section called“Removing numbering plan

interferences”

Changing the first number in the numbering plan

To change the first number in the numbering plan perform the following

procedure:

Procedure 54

Changing the first number in the numbering plan

- Pick up the handset of the administration telephone.

- Enter the administration Flexible Feature Code. (For some

countries, this value is listed in the “Flexible Feature Codes” on

page 324.)

3 	Enter the administration telephone password. (For some countries,

this value is listed in the, “Passwords and codes” on page 315.)

4 	You hear special dial tone and the prompt “TASK?” appears on the top

line of the character display.

5 	Press the asterisk (*) three times.

The second line of the character display reads “4 INSTALLATION

OPTIONS'”.

6	 Select “4 INSTALLATION OPTIONS” by entering the number “4”.

The character display reads:

1	 DEFAULT SETS

2 	NUMBERING PLAN

7	 Select “2 NUMBERING PLAN” by entering the number “2”.

The character display reads:

FIRST NUMBER (XXXX)?

8	 To create a new first number, enter the digits you want and press

the pound key.

9 	The number you enter as the first number in the numbering plan is

assigned to slot number one, unit zero of the main cabinet. The default

numbers assigned to the remaining card and unit combinations are

consecutive in the Option 11C system assuming that each slot

consists of up to 16 units.

Determining new extension numbers

If you wish to change the default numbering plan, and need to determine the

extension number that will be assigned to a specific telephone, perform the

following procedure:

Procedure 55

Determine the extension number

1	 Identify the line card to which the telephone is connected. Take the

card slot number associated with the card and subtract 1.

2	Multiply this number by 16.

- Add the first extension number in the new numbering plan.

4 	Identify the unit number the telephone is terminated on and add it to

the result obtained in step 3.

Example:

Assume the new numbering plan starts at extension number 4500. You wish

to know the extension number of the telephone connected to card 8, unit 12:

- Subtract 1 from the card slot number (8-1):

2 	Multiply by 16

- Add the first extension number: 						+4500

- Add the unit number: 							+ 12

Number assigned to card 8, unit 12 is 4624

Note: If you are using a three digit numbering plan, follow the same

Procedur

Using the Meridian Mail auto-configure feature

The Meridian Mail auto-configure feature automatically matches up

Meridian Mail mail boxes with telephone extensions. If you have the

Meridian Mail card option and you want to use the Meridian Mail

auto-configure feature, make sure of the following:

• The numbering plan is consecutive

• The first number in the Option 11 system and the Meridian Mail system

are the same.

Shifting the numbering plan to a new card slot

The first number in the numbering plan is assigned to slot 1, unit 0. You may

wish to shift the first number in the numbering plan to a different card slot,

since the TDS/DTR card is pre-programmed for slot 1.

The following procedure describes how to shift the first number in the

numbering plan to a specific card slot.

Procedure 56

Shift the first number in the numbering plan to a different card slot

1 Locate the first line card. Subtract 1 from the card slot number it is

housed in.

2 Multiply this number by 16.

3 Take the first number in the numbering plan and subtract the value

obtained in step 2.

Note: When you enter the value from step 3 in response to the prompt

“FIRST NUMBER (XXXX)?” on the administration telephone menu,

the number is shifted to the appropriate slot.

Example:

The first number in the numbering plan is 2200. If you want the first number

in the numbering plan to be 2300, and the first line card is located in slot 6,

you will carry out the following calculation:

1 	Locate the first line card. Subtract 1 from the card slot number it is

housed in.

6 - 1 = 5

2 	Multiply the number obtained in step 1 by 16:

X 16 = 80

3	Subtract this number from the first number in the numbering plan:

2300 - 80 = 2220

4 	Enter this value in response to the “FIRST NUMBER (2200)?” prompt

on the administration telephone:

FIRST NUMBER (2000)? 2220

Removing numbering plan interferences

This section contains various methods of removing any numbering plan

interferences that may occur.

Interference with Meridian Mail data

The Option 11 system is shipped with pre-programmed data to support a

Meridian Mail card option. The pre-programmed Meridian Mail data may

cause problems in the following situations:

• 	there is a numbering plan conflict with pre-programmed Meridian Mail

data and the programming required for another feature

• 	card slot 10 is required for another circuit card.

Obtaining the use of card slot 10

If card slot 10 is required for another circuit card, simply remove the six

Meridian Mail agents programmed for this card slot. To do this, load LD 11

on the TTY and respond to the system prompts as shown below. Repeat the

procedure once for each of the six units that you want to remove.

LD 11

REQ 	OUT 	<CR>

TYPE 	2008 	<CR>

TN 	10  XX 	<CR> “XX” = a value of 0,1,2,8,9, or 10

Obtaining the use of Meridian Mail ACD queues

The Meridian Mail feature also uses a number of ACD queues in the

Option 11 system. Refer to the section of this chapter called “Interference

with ACD queues” on page 393 for information on how to remove this data

from the system.

Interference with ACD queues

The Option 11C has pre-programmed ACD queues for Meridian Mail, the

Central Answering Position, and general purpose ACD. (For some countries,

pre-programmed ACD queues are listed in “Important extension numbers” on

page 318.)

To remove these values, use LD 23.

Note: Before you remove this data from the system, make sure that all

ACD agent information is removed from the queue

In LD 23, respond to the prompts as follows:

LD 23

REQ 	OUT

TYPE 	ACD

CUST	 0

ACDN XXXX “XXXX” = Value of ACD queue

Repeat for each ACD queue you wish to remove.

Interference with Call Park extension numbers

System call park extension numbers are pre-programmed for the Central

Answering Position. (For some countries, the default call part extensions are

listed in “Important extension numbers” on page 318.)

To remove this data from the system, load LD 50 and respond to the prompts as follows:

LD 50

REQ 	OUT

TYPE 	CPK

CUST	 0

SPDN XXXX “XXXX” = Value of Call Park extension

Repeat for each Call Park extension you wish to remove.

Interference with SDI ports

Five SDI/ESDI ports are pre-programmed on the Option 11 system. Ports 8

and 9 are associated with the Meridian Mail card option, and ports 0, 1 and 2

are associated with the NTDK20 Small System Controller (SSC) card. Port 0

is associated with SSC card and cannot be removed.

Any of the remaining ports can be removed using LD 17 as follows.

LD 17

REQ 	CHG

TYPE 	CFN

ADAN OUT TTY X “X” = the TTY that you are removing.

Interference with the SPRE code

If the pre-programmed SPRE code interferes with the programming required

for the Option 11 system, use LD 15 to remove it. (For some countries, the

default SPRE access code is listed in “First digits” on page 316.) For

customer 0 enter the following in response to the SPRE prompt:

LD 15

SPRE X	y 	“y” = the value of the SPRE code

Note: To define a new SPRE code, type a space and enter the new

number.

Changing or removing the pre-programmed night number

For some countries, the default value of the night number is listed in

“Important extension numbers” on page 318. If you wish to change or remove

the night number, use LD 15.

LD 15

REQ 	CHG	 CHG = Change

TYPE	 CDB	 CDB = Customer Data Block

CUST 	x	 “x” = Customer number

NIT1	 bbbb, 	or 	X	 aaaa 	“bbbb” = the new extension number (DN)

Or

“aaaa” = the current night number

(the night number is now removed).

Interference with the attendant extension number

You cannot remove the attendant extension number entirely from the

Option 11 system data; you can only replace it with another number.

LD 15

REQ CHG 	CHG = Change

TYPE	CDB	CDB = Customer Data Block

CUST	 x	 “x” = Customer number

.

ATDN xxxx “xxxx” = the new extension number

Interference with Flexible Feature Codes

If the pre-programmed Flexible Feature Codes interfere with the

programming required for the Option 11 system, use LD 57 to remove the

data from the system. (For some countries, default FFCs are listed in

“Flexible Feature Codes” on page 324.)

LD 57

To change one or more access codes, type in the following commands:

REQ	 CHG, END 		Change or end

TYPE	 FFC 			FFC = Flexible feature codes

CUST 	0-31			 Customer Number

FFCT 	YES,(NO) 		FFC Confirmation tone

CODE 	aaaa 			Enter access code prompt (aaaa)

AAAA 	xx 			Enter the new access code prompt

(AAAA)

CODE	 <CR>			 Return to REQ

REQ 	END			 End program

To remove one access code, type in the following commands:

REQ 	OUT 			Action request

TYPE	 FFC 			FFC = Flexible feature codes

CUST	 0-31			 Customer Number

FFCT 	YES,(NO) 		FFC Confirmation tone

ALL 	NO			 Remove specific access code

CODE	 aaaa 			Enter access code prompt (aaaa)

AAAA	 xx 			Enter the new access code prompt

(AAAA)

CODE 	<CR>			 Return to REQ

REQ 	END			 End program

To remove all access codes, type in the following commands:

REQ 	OUT 			Action request

TYPE 	FFC			 FFC = Flexible feature codes

CUST 	0-31 			Customer Number

FFCT 	YES,(NO)		 FFC Confirmation tone

ALL 	YES 			Remove all access codes

CODE	<CR> 			Return to REQ

REQ 	END			 End program

Creating, changing, and removing model telephones

If you need a model telephone different from the default models provided in

Option 11C software, use overlays to design your own models using the

information in the following table:

If you require additional assistance when creating your own model telephones

and trunks, refer to the Option 11C Software guides that are shipped with

every system.

Creating analog telephone models

The following information must be entered in LD 10 in order to create an

analogue (500/2500- type) telephone model:

LD 10

REQ	 NEW

TYPE 	500 M

MODL	 YYY 		“YYY” = the model number of the telephone that you are

Creating

Enter responses to the remaining prompts in order to complete the new model.

(Refer to the Option 11C Software guides for a complete list of prompts and

possible responses

Modifying analog model telephones

If you want to modify an analog telephone, use LD 10 and type in the

following commands on the TTY:

LD 10

REQ	 CHG

TYPE 	500 M

MODL 	YYY		 “YYY” = the model number of the telephone that you are

Modifying

For more information about LD 10 and its associated prompts and

commands, refer to the Option 11 Software Guides that are shipped with the

system.

Creating digital model telephones

When you create model telephones for digital telephones, you must program

key 0 with a function that can act as a prime extension number or its

equivalent. This includes telephones that are programmed to have Single Call

Ringing (SCR), Multiple Call Ringing (MCR), Single Call Non-Ringing

(SCN), Multiple Call Non-Ringing (MCN), and Automatic Call Distribution

(ACD).

For ACD telephones, the model telephone defines the ACD queue. To define

the extension number, you must enter the ID number of the agent or the

Central Answering Position.

The following information must be entered in LD 11 in order to create a

digital telephone model:

LD 11

REQ 	NEW

TYPE 	2XXX M or 3000 M 		“XXX” = allowed telephone typesMODL 	YYY 					“YYY” = the model number of the lephone                          that you are creating (between 1 - 127)

Enter the responses to the remaining prompts in order to complete the new

model. (Refer to the Option 11C Software guides for a complete list of

prompts and possible responses).

Modifying digital model telephones

If you want to modify a digital telephone, load LD 11 and type in the

following commands on the TTY:

LD 11

REQ	 CHG

TYPE	 2XXX M or 3000 M	 “XXX” = allowed telephone types

MODL	 YYY			 “YYY” = the model number of the telephone that you are

Modifying

For more information about LD 11 and its associated prompts and

commands, refer to the Option 11C Software guides that are shipped with the

system

Printing model information

To print information about model telephones, use LD 20.

Analog telephones

LD 20

REQ	 PRT

TYPE 	500 M

MODL 	YYY 			“YYY” = the telephone model number.

If you want to print all of the models for

this telephone type, leave this value blank.

Digital telephones

Note: To print M3000 telephone models, you must enter “TYPE 3000

M”. If you enter “TYPE 2XXX M”, you print all the Meridian Digital

Telephone models except the M3000.

LD 20

REQ	 PRT

TYPE 	2XXX M or 3000 M	 “XXX” = the telephone type

MODL	 YYY			 “YYY” = the telephone model number

(between 1 - 127) If you want to print all

of the models for this telephone type, leave

this value blank.

Removing model telephones

Use LD 10 to remove analog telephone models and LD 11 to remove digital

telephone models. Respond as follows to the prompts in either overlay:

LD 10 or LD 11

REQ OUT

TYPE 	XXXX M 		“XXXX” = telephone type for the model

you are removing (Example: 500, 2006,

2317, 2216, 3000, etc.)

CUST 	0

MODL 	YYY 			“YYY” = the model number associated

with the telephone type you are removing

(Valid range is 1-127).

Creating model trunks and changing route access codes

If you need a trunk that is different from the default models provided in

Option 11 software, use overlay 14 to design your own models. Route access

codes are changed using the administration telephone.

If you require additional assistance when creating your own model telephones

and trunks, refer to the Option 11 Software guides that are shipped with every

system.

Creating model trunks

To create a new model trunk, load LD 14 and type in the following commands

on the TTY:

LD 14

REQ	 NEW

TYPE	 aaa M			 “aaa” = the type of trunk that you are

creating (TIE, COT, WATS, and so on).

MODL 	YYY 		“YYY” = the model number of the trunk that you are creating

(between 1 - 127)

XTRK 	XUT, XEM, XCOT, 	This prompt only appears when you

XDID define the first model in a group. Each

group consists of 16 consecutive model

definitions. (Refer to the group boundaries

listed below.) Once the first model in a

group is defined, the remaining model

numbers in the group(s) are assumed to

be of the same type.

Group boundaries are as follows:

1-15, 16-31, 32-47, 48-63, 64-79. 80-95, 96-111, 112-127.

Examples:

If, for example, you define a model trunk as XUT, with a model number of

12, all models in the group 1-15 will automatically be XUT models. If you

define another model trunk as an XEM, with a model number of 33, all

models in the group 32-47 will automatically be XEM models.

Note: When you create trunk models you are not prompted for the trunk

route or member number. This information is defined by using the

administration telephone to program the XUT or XEM circuit card.

For more information about LD 14 and its associated prompts and

commands, refer to the Option 11 Software guides that are shipped with the

system.

Modifying model trunks

If you want to modify a trunk, load LD 14 and type in the following

commands on the TTY:

LD 14

REQ	 CHG

TYPE 	aaa M 			“aaa” = the type of trunk that you are

Modifying

MODL 	YYY			 “YYY” = the model number of the trunk

that you are modifying (between 1 - 127)

Removing model trunks

To remove a trunk model from the system, load LD 14 and respond to the

prompts as follows:

LD 14

REQ 	OUT

TYPE 	XXX M			 “XXX” = the trunk type of the model that

you are removing (Examples: COT, TIE,

DID, FEX, WAT, etc.)

CUST 	0

MODL	 YYY			 “YYY” = the model number associated

with the trunk type you are removing

(Valid range is 1-127).

Printing model information

To print information about model trunks, load LD 20 and type in the

following commands on the TTY:

LD 20

REQ 	PRT

TYPE	 TTTT M		 “TTTT” = the trunk type

MODL	 YYY 			“YYY” = the model number. If you want to print all of the

models for this trunk type, leave this value blank.

Changing a route access code

Use this procedure to change a route access code.

Procedure 57

Changing a route access code

- Pick up the handset of the administration telephone.

2	 Enter the administration Flexible Feature Code to access the

administration menu. (For some countries, this value is listed in

“Flexible Feature Codes” on page 324.)

3 	Enter the default administration telephone password. (For some

countries, this value is listed in “Passwords and codes” on

page 315.)

You hear special dial tone and the prompt

“TASK?”

appears on the top line of the character display. If you press the

asterisk, “2 CHANGE ROUTE ACCESS” appears on the second line

of the character display.

4 	Select “2 CHANGE ROUTE ACCESS” by entering the

number “2”.

The prompt

“ROUTE ACCESS?”

appears on the character display.

5 	Enter the access code of the route to be modified and press the

pound key (#).

The prompt

“NEW ACCESS CODE?”

asks you for a new access code for the route.

6 	Enter the new access code and press the pound key.

The display shows

“CODE CHANGED”.

After a delay of approximately 4 seconds you hear special dial tone

and the sequence is repeated when the prompt

“ROUTE ACCESS?”

appears.

OR

If the route access code is not available for use, you hear overflow

tone.

The display shows

“USED, ROUTE ACCESS?”

and you must repeat this step.

You must enter a different access code. This sequence continues

until you enter an appropriate new access code.

7 	Terminate the sequence by hanging up the telephone, or

Repeat the sequence by going through the steps again.

Cambiar El nombre a una extensión:

LD 95 
CND000 
MEM AVAIL: (U/P): 472449 USED: 117374 TOT: 589823 
DISK RECS AVAIL: 426 
REQ chg------------------cambio 
TYPE name---------------nombre 
CUST 0-------------------cust, por lo regular siempre es 0  
CPND_LANG--------------enter 
DIG ----------------------enter 
DN 2696-----------------numero de la extension a la que le vas a cambiar el nombre 
NAME SITE---------------nombre que le vas a poner a la extension 
DISPLAY_FMT------------enter 
DN -----------------------enter 
DCNO---------------------enter 
MEM AVAIL: (U/P): 472449 USED: 117374 TOT: 589823 
DISK RECS AVAIL: 426 
REQ 
Es una impresión que saque de un cliente, este es el procedimiento para cambiar un nombre de extensión, si vas a dar de alta un nombre nuevo, le tienes que poner en la parte del REQ new y lo demás es lo mismo.